LABORATORY REPORT

Arduino-Based LED and Motor Control

| Student Name | Hoor Aman |
| --- | --- |
| Department | Computer Engineering |
| Course | Embedded Systems |
| Date | May 03, 2026 |

# 1. OBJECTIVE

The objective of this lab is to design and implement a Bluetooth-controlled system using Arduino that integrates multiple components including an LED controlled via a relay module and a DC motor controlled via an L298N motor driver. The system demonstrates understanding of embedded systems principles including digital I/O control, serial communication, Bluetooth integration, and motor control techniques.

# 2. INTRODUCTION

This laboratory project integrates several key embedded systems concepts:

• Relay Control: Using digital pins to control a relay module that switches an LED on/off
• Motor Control: Implementing directional control of a DC motor using an L298N motor driver
• Wireless Communication: Utilizing Bluetooth serial communication for remote control
• Arduino Programming: Developing efficient embedded C code with command parsing and state management

# 3. HARDWARE COMPONENTS

| Component | Specifications |
| --- | --- |
| Microcontroller | Arduino Uno (ATmega328P) |
| Bluetooth Module | HC-05 (Software Serial Pins 10, 11) |
| Relay Module | 5V Relay (Connected to Pin 7) |
| Motor Driver | L298N Dual H-Bridge (IN1=Pin 4, IN2=Pin 3) |
| Actuators | LED (via relay), DC Motor |

# 4. CIRCUIT DESIGN & CONNECTIONS

The circuit diagram showing all connections is displayed below:

# 5. PIN CONFIGURATION

| Arduino Pin | Component | Function |
| --- | --- | --- |
| Pin 3 | L298N IN2 | Motor Backward Control |
| Pin 4 | L298N IN1 | Motor Forward Control |
| Pin 7 | Relay Module | LED Control (Active LOW) |
| Pin 10 | HC-05 RX | Bluetooth Receive |
| Pin 11 | HC-05 TX | Bluetooth Transmit |
| Serial | USB/UART | Debug Communication |

# 6. SOFTWARE IMPLEMENTATION

6.1 Program Overview

The embedded C code running on the Arduino implements a command parser that responds to wireless Bluetooth commands. The main control loop continuously monitors both the Bluetooth Serial and USB Serial ports for incoming commands and executes the corresponding actions.

6.2 Key Functions

handleCommand(): Processes incoming characters and executes appropriate actions

motorForward(): Activates motor in forward direction (IN1=HIGH, IN2=LOW)

motorBackward(): Activates motor in backward direction (IN1=LOW, IN2=HIGH)

stopMotor(): Halts motor operation (IN1=LOW, IN2=LOW)

sendStatus(): Reports current state of LED and motor

printMenu(): Displays available commands to the user

6.3 Command Structure

| Command | Action | Type |
| --- | --- | --- |
| 1 | LED ON | LED Control |
| 0 | LED OFF | LED Control |
| t / T | Toggle LED | LED Control |
| f / F | Motor Forward | Motor Control |
| b / B | Motor Backward | Motor Control |
| s / S | Motor Stop | Motor Control |
| ? | Display Status | System |
| m / M | Display Menu | System |

# 7. TASK 1: LED CONTROL

The LED is controlled via a 5V relay module connected to Arduino Pin 7. The relay provides electrical isolation and allows switching of higher-power circuits. The implementation uses active-LOW logic, where setting the pin LOW activates the relay and turns the LED ON.

7.1 LED Control Implementation

Figures 2-4: LED Control Testing - ON/OFF/Toggle States

7.2 Results & Observations

• The relay responds immediately to digital commands

• LED activation is reliable and stable with no flickering

• Wireless Bluetooth control works seamlessly

• The active-LOW logic properly inverts the signal

• Status feedback confirms correct state changes

# 8. TASK 2: MOTOR CONTROL

The DC motor is controlled using an L298N H-Bridge motor driver, which allows bidirectional control (forward, backward, and stop). Pin 4 controls the direction via IN1, and Pin 3 controls direction via IN2. The L298N is permanently enabled (no PWM speed control in this configuration) for constant-speed operation.

8.1 Motor Control States

| State | IN1 (Pin 4) | IN2 (Pin 3) |
| --- | --- | --- |
| Forward | HIGH | LOW |
| Backward | LOW | HIGH |
| Stop | LOW | LOW |

8.2 Motor Control Testing

Figures 5-8: Motor Control Testing - Forward/Backward/Stop States

8.3 Motor Performance Results

• Forward rotation executes smoothly with immediate response

• Backward rotation reverses motor direction reliably

• Stop command halts motor operation instantly

• No mechanical binding or stalling observed

• H-Bridge properly manages back-EMF from motor

• Current limiting provides safe operation

9. BLUETOOTH COMMUNICATION & TESTING

The HC-05 Bluetooth module enables wireless control via smartphone or serial terminal emulator. SoftwareSerial on pins 10-11 handles the Bluetooth communication at 9600 baud.

9.1 Bluetooth Testing Setup

Figures 9-11: Bluetooth Communication and Testing

9.2 Communication Results

• Bluetooth module pairs successfully with external devices

• Commands received via wireless link execute correctly

• Status messages display properly on remote terminal

• Menu is accessible and user-friendly

• No data corruption observed during extended testing

• Baud rate stability maintained throughout operation

# 10. COMPLETE ARDUINO CODE

#include <SoftwareSerial.h>

SoftwareSerial BTSerial(10, 11); // RX=10, TX=11

// ── Pin definitions ──────────────────────────
const int RELAY_PIN = 7;   // Relay → LED
const int IN1       = 4;   // L298N IN1
const int IN2       = 3;   // L298N IN2

bool ledState   = false;
bool motorState = false;

// ── Setup ────────────────────────────────────
void setup() {
  pinMode(RELAY_PIN, OUTPUT);
  pinMode(IN1, OUTPUT);
  pinMode(IN2, OUTPUT);

  digitalWrite(RELAY_PIN, HIGH); // LED OFF at start
  stopMotor();                   // Motor OFF at start

  Serial.begin(9600);
  BTSerial.begin(9600);
  printMenu();
}

// ── Loop ─────────────────────────────────────
void loop() {
  if (BTSerial.available()) {
    char cmd = (char)BTSerial.read();
    Serial.print("BT CMD: ");
    Serial.println(cmd);
    handleCommand(cmd);
  }
  if (Serial.available()) {
    char cmd = (char)Serial.read();
    handleCommand(cmd);
  }
}

// ── Command Handler ───────────────────────────
void handleCommand(char cmd) {
  switch (cmd) {
    case '1':
      digitalWrite(RELAY_PIN, LOW);
      ledState = true;
      BTSerial.println("LED ON");
      Serial.println("LED ON");
      break;
    case '0':
      digitalWrite(RELAY_PIN, HIGH);
      ledState = false;
      BTSerial.println("LED OFF");
      Serial.println("LED OFF");
      break;
    case 't': case 'T':
      ledState = !ledState;
      digitalWrite(RELAY_PIN, ledState ? LOW : HIGH);
      BTSerial.println(ledState ? "LED Toggled ON" : "LED Toggled OFF");
      break;
    case 'f': case 'F':
      motorForward();
      BTSerial.println("Motor: FORWARD");
      Serial.println("Motor: FORWARD");
      break;
    case 'b': case 'B':
      motorBackward();
      BTSerial.println("Motor: BACKWARD");
      Serial.println("Motor: BACKWARD");
      break;
    case 's': case 'S':
      stopMotor();
      BTSerial.println("Motor: STOPPED");
      Serial.println("Motor: STOPPED");
      break;
    case '?':
      sendStatus();
      break;
    case 'm': case 'M':
      printMenu();
      break;
    default:
      BTSerial.println("Unknown! Send 'm' for menu.");
      break;
  }
}

// ── Motor Functions ───────────────────────────
void motorForward() {
  digitalWrite(IN1, HIGH);
  digitalWrite(IN2, LOW);
  motorState = true;
}

void motorBackward() {
  digitalWrite(IN1, LOW);
  digitalWrite(IN2, HIGH);
  motorState = true;
}

void stopMotor() {
  digitalWrite(IN1, LOW);
  digitalWrite(IN2, LOW);
  motorState = false;
}

// ── Status ────────────────────────────────────
void sendStatus() {
  BTSerial.println("=== STATUS ===");
  BTSerial.print("LED   : ");
  BTSerial.println(ledState ? "ON" : "OFF");
  BTSerial.print("Motor : ");
  BTSerial.println(motorState ? "RUNNING" : "STOPPED");
  BTSerial.println("==============");
}

// ── Menu ──────────────────────────────────────
void printMenu() {
  String m = "\n====== CONTROL MENU ======\n";
  m += "LED:\n";
  m += "  1 = LED ON\n";
  m += "  0 = LED OFF\n";
  m += "  t = Toggle LED\n";
  m += "MOTOR:\n";
  m += "  f = Forward\n";
  m += "  b = Backward\n";
  m += "  s = Stop\n";
  m += "OTHER:\n";
  m += "  ? = Status\n";
  m += "  m = This menu\n";
  m += "==========================\n";
  Serial.print(m);
  BTSerial.print(m);
}

# 11. CONCLUSION

This laboratory project successfully demonstrated the integration of multiple embedded systems components into a unified wireless control platform. The implementation achieved all objectives:

Key Accomplishments:
• Successfully implemented Bluetooth wireless control using HC-05 module
• LED control via relay module with proper electrical isolation
• DC motor directional control using L298N H-Bridge driver
• Robust command parsing and state management
• Reliable serial communication at 9600 baud
• User-friendly menu system with status feedback

The system operates reliably with low latency response to wireless commands. The modular architecture allows for future enhancements such as:
• PWM speed control for variable motor speeds
• Multiple motor control
• Sensor feedback integration
• Mobile app development for enhanced user interface
• Power management optimization

This project effectively demonstrates competency in embedded C programming, hardware interfacing, wireless communication protocols, and system integration - all critical skills for embedded systems engineers.

Final Hardware Assembly:

Figures 12-14: Final Hardware Assembly and Integration